principle of superposition

As there will usually be many of pressure drops occurring throughout the prediction period, it is necessary to analyze the procedure to be used where these multiple pressure drops are present.

Figure 1 Shown boundary pressure versus time.

Consider figure above, which illustrates the decline in the boundary pressure as a function of time for a radial reservoir-aquifer system. If the boundary pressure in the reservoir shown in previous figure is suddenly reduced at time t, from pi to p1, a pressure drops of (pi – p1) will be imposed across the aquifer. Water will continue to expand and the new reduced pressure will continue to move outward into the aquifer. Given a sufficient length of time the pressure at the outer edge of the aquifer will finally be reduced to p1.

If some time after the boundary pressure has been reduced to p1, a second pressure p2 is suddenly imposed at the boundary, and a new pressure wave will begin moving outward into the aquifer. This new pressure wave will also cause water expansion and therefore encroachment into the reservoir. This new pressure drop, however, will not be pi – p2, but will be p1 – p2. This second pressure wave will be moving behind the first pressure wave. Just ahead of the second pressure wave will be the pressure at the end of the first pressure drop, p1.

Since these pressure waves are assumed to occur at different times, they are entirely independent of each other. Thus, water expansion will continue to take place as a result of the first pressure drop, even though additional water influx is also taking place as a result of one or more later pressure drops. This is essentially an application of the principle of superposition.

The superposition principle asserts that the total pressure change at a point in the reservoir is a linear sum of the changes in pressure due to each well in the reservoir. The superposition principle implies that a pressure disturbance will propagate through the reservoir even if the source of the disturbance changes or disappears.

In order to determine the total water influx into a reservoir at any given time, it is necessary to determine the water influx as a result of each successive pressure drop that has been imposed on the reservoir and aquifer.

In calculating cumulative water influx into a reservoir at successive intervals, it is necessary to calculate the total water influx from the beginning. This is required because of the different times during which the various pressure drops have been effective.

The van Everdingen-Hurst computational steps for determining the water influx are summarized below in conjunction with following figure:

Figure 2 Illustration of the superposition concept.

Step 1. Assume that the boundary pressure has declined from its initial value of pi to p1 after t1 days. To determine the cumulative water influx in response to this first pressure drop, Δp1 = pi – p1 can be simply calculated from following equation:

Where We is the cumulative water influx due to the first pressure drop Δp1. The dimensionless water influx (WeD)t1 is evaluated by calculating the dimensionless time at t1 days. This simple calculation step is shown in section A of previous figure.

Step 2. Let the boundary pressure decline again to p2 after t2 days with a pressure drop of Δp2 = p1 – p2. The cumulative (total) water influx after t1 days will result from the first pressure drop Δp1 and the second pressure drop Δp2, or:

Where

The above relationships indicate that the effect of the first pressure drop Δp1 will continue for the entire time t2, while the effect of the second pressure drop will continue only for (t2 – t1) days as shown in section B of Figure 2.

Step 3. A third pressure drop of Δp3 = p2 – p3 would cause an additional water influx as illustrated in section C of Figure 2. The cumulative (total) cumulative water influx can then be calculated from:

Where

The van Everdingen-Hurst water influx relationship can then be expressed in a more generalized form as:

The authors also suggested that instead of using the entire pressure drop for the first period, a better approximation is to consider that one-half of the pressure drop, ½ (pi – p1), is effective during the entire first period. For the second period, the effective pressure drop then is one-half of the pressure drop during the first period, ½ (pi – p2) which simplifies to:

Figure 3 Sketch showing the use of step pressure to approximate the pressure-time curve.

Similarly, the effective pressure drop for use in the calculations for the third period would be one-half of the pressure drop during the second period, ½ (p1 – p2), plus one-half of the pressure drop during the third period, ½ (p2 – p3), which simplifies to ½ (p1 – p3). The time intervals must all be equal in order to preserve the accuracy of these modifications. As shown below the effective pressure drops for use in the calculations for the next periods:

4.4.2 Bottom-Water Drive

The van Everdingen and Hurst model discussed in the previous lectures is based on the radial diffusivity equation written without a term describing vertical flow from the aquifer.

correctly noted that in many cases reservoirs are situated on top of an aquifer with a continuous horizontal interface between the reservoir fluid and the aquifer water and with a significant aquifer thickness.  that in such situations significant bottom-water drive would occur.

The proposed solution technique (van Everdingen and Hurst model ), however, is not adequate to describe the vertical water encroachment in bottom-water-drive system. Coats (1962) presented a mathematical model that takes into account the vertical flow effects

Figure 4 Sketch of bottom-water drive reservoir.

Coats and, later, Allard and Chen added a term to Eq. below to yield the following:

to be as following

They suggested that it is possible to derive a general solution that is applicable to a variety of systems in terms of the dimension-less time tD, dimensionless radius rD, and a newly introduced dimensionless variable ZD.

Where:

ZD = dimensionless vertical distance.

h = aquifer thickness, ft.

where

Fk is the ratio of vertical to horizontal permeability,

or:

Fk = KvKh

where

kv = vertical permeability

kh = horizontal permeability

Using the definitions of dimensionless time, radius, and pressure and introducing a second dimensionless distance, zD, last Eq. becomes as following:

The authors developed a solution to the bottom-water influx that is comparable in form with that of van Everdingen and Hurst.

They defined the water influx constant B identical to that of Equation in van Everdingen and Hurst ,

where

We = cumulative water influx, bbl

B = water influx constant, bbl/psi

Δp = pressure drop at the boundary, psi

WeD = dimensionless water influx