Porosity

From the petrophysical properties standpoint, one of the most important rock properties is porosity. Porosity is defined as a measure of the capacity of reservoir rocks to contain or store fluids.

Engineering Classification of Porosity

- total (absolute)

- effective.

Geological Classification of Porosity

- (primary or matrix porosity)

- (secondary or induced porosity).

∅=Vb-VgrVb=VpVb

where

∅ = porosity, fraction

Vb = bulk volume of the reservoir rock

Vgr = grain volume

Vp = pore volume

Example

Calculate the porosity for cubic packing which is containing eight balls with diameter D, with sides equal to 2D as shown in below.

(4/3)×π(D/2)3=0.5236D3×8=4.1888D3

؞ ∅=8D3-4.1888D38D3×100=47.64 	per cent

Problem

Calculate the porosity for rhombic packing which is containing eight balls with diameter D and 45° angle, with sides equal to 2D as shown in below.

V bulk =2D×2D×2D×sin 45°=8D3×0.7071=5.65658 D3

Vgrains = 8×π×4/3×D38=4.1888 D3

∅=5.65658 D3-4.1888 D35.65658 D3×100=25.951

Example

A clean and dry core sample weighing 425 g was 100% saturated with 1.07 specific gravity (γ) brine. The new weight is 453 g. The core sample is 12 cm in length and 4 cm in diameter. Calculate the porosity of the rock sample.

Answer

The bulk volume is

Vb=πr2×l

Vb=π22×12=150.8 cm3

The pore volume is

Vp=1γVwet-Vdry=453-4251.07=26.17cm3

∅=VpVb=26.17150.8=0.173 or 17.3%

Example

An irregular piece of sandstone is 35.25 grams in mass. When coated with varnish, Its mass increased to 36.55 grams. Compute the rock porosity if coated sample  displaces 15.7 ml of water when fully submerged.

ρg=2.65gmcm3, ρw=1.00gmcm3 ,ρv=1.80gmcm3

Answer

vg=35.252.65=13.30 cm3

vv=(36.55-35.25)1.80=0.72 cm3

vb=15.7-0.72=14.98 cm3

Ø=(14.98-13.3)14.98=0.112 or 11.2%

Some properties of the porous medium do not have the same value in different directions (that means Anisotropy). In an anisotropic porous medium, the permeability, and formation resistivity factor depend on the direction. In the most general case these properties are function of both location in the medium and orientation.

An averaging techniques are expressed mathematically in the following forms:

Arithmetic average φ = Σni φi /n

Thickness-weighted average φ = Σni φi hi / Σnihi

Areal-weighted average φ = Σniφi Ai /ΣAi

Volumetric-weighted average φ = Σniφi Ai hi / ΣniAi hi

where

n = total number of core samples

hi = thickness of core sample i or reservoir area i

φi = porosity of core sample i or reservoir area i

Ai = reservoir area i

Example

Calculate the arithmetic average and thickness-weighted average from the following measurements:

Example 3.5

Given a stacked reservoir with three sand of different reservoir pay thickness in the figure below. The reservoir structure is 710 acres. The initial oil formation volume factor from PVT laboratory analysis is given as 1.3251 rb/stb.

Average water saturated =33%

The petrophysical properties are stated on each sand in the figure.

Calculate the stock tank oil initially in place for this stacked reservoir.

مصدر Reservoir Engineering Fundamentals and Applications

صفحة 125 حاسبة

Original oil in place

The basis for the volumetric method for estimating the original hydrocarbon in place is the estimation of total HCPV of the reservoir. It is a product of the delineated area of the reservoir, net thickness of the pay zone, rock porosity, and the initial saturation of oil or gas or both. The following terminologies associated with the volumetric estimates are noteworthy.

Bulk volume of the reservoir, BV =  Reservoir area ×thickness

Pore volume of the reservoir, PV =Reservoir area ×  thickness× porosity

Hydrocarbon pore volume, HCPV= Reservoir area ×thickness ×porosity ×saturation of hydrocarbon fluid

The general equation for estimating the OOIP in conventional oil reservoirs is as follows:

where

A = areal extent of the reservoir, acres;

h = net thickness of the oil bearing formation, ft.;

ø = average porosity, fraction;

Swi = initial water saturation, fraction;

Boi = oil formation volume factor, rb/STB; STB = stock-tank barrel.

Original oil and gas in place

Bgi = gas formation volume factor is rb/SCF.

Example 12.1

Calculate the pore volume, OOIP.

Reservoir drainage area = 600 acres

Average oil zone net thickness = 28 ft.

Average porosity = 0.205 fraction

Initial water saturation (fraction) = 0.20

Initial oil formation volume factor = 1.56 rb/STB

Solution

bulk volume of the reservoir:

BV = 7758×  600 ×28 =130.334,400 MMbbl

pure volume of the reservoir:

PV = 7758×  600 ×28 ×0.205= 26.718,552 MMbbl

Note that 1 MMSTB = 1×106 STB of oil.

The volume of OOIP:

OOPI=7758bblacre-ft×600 acre×28 ft×0.205(1-0.2)1.5613 bblSTB=13.690,412 MMSTB

المصدر

Reservoir Engineering The Fundamentals, Simulation, and Management of Conventional and Unconventional Recoveries

صفحة 199 حاسبة