Example:

The Virginia Hills Beaverhill Lake field is a volumetric undersaturated reservoir. Volumetric calculations indicate the reservoir contains 270.6 MMSTB of oil initially in place. The initial reservoir pressure is 3685 psi. The following additional data are available:

Swi = 24%

Cw = 3.62×10-6 psi-1

Cf = 4.95 ×10-6 psi-1

Bw = 1.0 bbl/STB

Pb = 1500 psi

The field production and PVT data are summarized below:

Calculate the initial oil in place by using the MBE and compare with the volumetric estimate of N.

Solution:

Step 1.

Calculate the initial water and rock expansion term Ew,f from 𝑒𝑞:

Step 2.

Construct the following table:

Underground withdrawal F vs (Eo + Ew,f)

Step 3.

Plot the underground withdrawal term F against the expansion term (Eo + Ew,f) on a Cartesian scale, as shown in Figure.

Step 4.

Draw the best straight line through the points and determine the slope of the line and the volume of the active initial oil in place as:

N≅257 MMSTB

It should be noted that the value of the initial oil in place as determined from the MBE is referred to as the effective or active initial oil in place. This value is usually smaller than that of the volumetric estimate due to oil being trapped in undrained fault compartments or low-permeability regions of the reservoir.

Reservoir Engineering Handbook, Third Edition by Tarek Ahmed PhD PE (z-lib.org)

Problem:

An undersaturated reservoir producing above the bubble point had an initial pressure of 5000 psia, at which pressure the oil formation volume factor was 1.510 bbl/stb.

When the pressure dropped to 4600 psia owing to the production of 100,000 stb of oil, the oil formation volume factor is 1.520 bbl/stb, the connate water saturation is 23%, water compressibility is 3.7 ×10-6 psi-1, rock compressibility is 3.5  3.7 ×10-6 psi-1 and average porosity of 21%.

I. Determine the oil compressibility.

II. Determine the effective oil compressibility

III. Assuming a volumetric reservoir, calculate the oil in place

IV. Determine the recovery factor at 4600 psia

V. After a thorough analysis, the calculated initial oil place was 9.6 MMstb.

Determine the water influx at 4600 psi after cumulative water production of

825.92 stb.

Problem:

An undersaturated reservoir producing above the bubble point

Pi=5000 psi,

Boi= 1.510 bbl/stb.

P= 4600 psi

Bo= 1.520 bbl/stb

Np= 100,000 stb

Sw= 23%

Cw= 3.7 ×10-6 psi-1

Cf= 3.5  3.7 ×10-6 psi-1

Ø=of 21%.

I. Determine Co.

II. Determine Ceff

III. Assuming a volumetric reservoir, calculate the oil in place

IV. After a thorough analysis, the calculated initial oil place was 9.6 MMstb.

Determine We= 4600 psi after Wp=825.92 stb.

V. Determine the recovery factor at 4600 psi.

Solution:

Oil compressibility

Calculation of oil in place

Representation of Material Balance Equation For Undersaturated Reservoir (P > Pb) with Water Influx

Then oil in place(volumetric):

Water influx:

The recovery factor at 4600 psi without water influx:

Reservoir Engineering Fundamentals and Applications

صفحة 239 حاسبة