Example

Calculate Datum Level Pressures, Pressure Gradients, and Reservoir Flow from Static Pressure Measurements in Wells as shown below:

Data given

Distance between wells true stratum thickness = 20 ft

Dip of stratum between wells = 8° 37′

Reservoir datum level = 7600 ft subsea

Reservoir fluid specific gravity = 0.693 (water = 1.00)

ϕ = 20%

Reservoir permeability = 145 md

Reservoir fluid viscosity = 0.32 cp

Well number 1 static pressure = 3400 psia at 7720 ft subsea

Well number 2 static pressure = 3380 psia at 7520 ft subsea

نظرا للإشارة الى ان الضغط هو مستقر كما مشار اليه Static Pressure Measurements فيتم التعامل مع المائع الذي يجري في الوسط المسامي وهو النفط كانه مائع غير قابل للانضغاط Incompressible وليس منضغط قليلا  Slightly compressible ذلك بسبب ثبوت الضغط والذي يعطي انطباع على ان الانضغاطية للمائع تبقى ثابته أي يتصرف كمائع غير قابل للانضغاط

Solution:

calculation for well No.1

φi=Pi-0.433×ɤ×Δzi

φi=3400psi-0.433psi/ft×0.693×(7720-7600)ft=3364 psi

Reservoir fluid gradient =

Reservoir fluid specific gravity × Hydraulic gradient fresh water =

0.693 × 0.433 psi/ft = 0.300 psi/ft

calculation for well No.2

φi=Pi+0.433×ɤ×Δzi

φi=3380+0.433×0.693×(7600-7520)=3404 psi

The difference of 40 psi indicates that fluid is moving downdip, from well 2 to well 1.

Ѳ=8° 37′

minute/60=degree

37/60=0.616666

Ѳ=8.616666°

cosѲ=xr

r=1320/cos 8.616666=1335 ft

The average effective gradient is

40/1335 = 0.030 psi/ft,

where 1335 is the distance along the stratum between the wells.

The velocity then is

Average effective gradient

v = 0.001127 × (Reservoir permeability / Reservoir fluid viscosity) × Average effective gradient

V=qA =0.001127×A× KM×A×(Φ1-Φ2)L

Apparent fluid velocity v =  0.001127 × 145×0.0300.32 =0.0153 2bbl/day/ft2

Apparent fluid velocity v = 0.0153 2bbl/day/ft2×5.615 ft3/bbl

Apparent fluid velocity v =0.086  ftday

Actual fluid velocity V = qØ×A =  VØ

VØ=0.01530.2 = 0.0765 bbl/day/ft2×5.615 ft3/bbl 
Actual fluid velocity V =0.429 ftday