In petroleum engineering, dealing mathematically with more than one term for porosity.

- Absolute porosity

- Effective porosity

- Residual porosity.

Example 1

Unit cells of two of the packings studied, cubical packing and Rhombohedral as shown in below.

The porosity for cubical packing is 47.6 per cent. Considering cubic packing, the porosity can be calculated as follows:

Example 2

اعطي

Calculate the porosity for cubic packing which is containing eight balls with diameter D, with sides equal to 2D as shown in below.

∅=Vb-VmVb=VpVb

where

φ = porosity, fraction

Vb = bulk volume of the reservoir rock

Vgr = grain volume

Vp = pore volume

Then

Vb = bulk volume of the reservoir rock = (side length)3 = (2r)3 = 8r3

Vgr = grain volume

Vgr = volume of ball ×number of balls = (4/3)×π(r)3=(4/3)×π r3×8=4.1888r3

؞ ∅=8r3-4.1888r38r3×100=47.64 	per cent

Or

Vb = bulk volume of the reservoir rock = (side length)3 = (2D)3 = 8D3

Vgr = grain volume

Vgr = volume of ball ×number of balls = (4/3)×π(D/2)3×8

Vgr = 0.523598 (D)3×8=4.1888r3

؞ ∅=8D3-4.1888D38D3×100=47.64 	per cent

Homework:

Calculate the porosity for rhombic packing which is containing eight balls with diameter D and 45° angle, with sides equal to 2D as shown in below.

Solution:

Vbulk =2D×2D×2D×sin 45°=8D3×0.7071=5.65658 D3

Vgrains = 8×π×4/3×D38=4.1888 D3

∅=5.65658 D3-4.1888 D35.65658 D3×100=25.951%

Example 3

Effective porosity by the saturation method.

A = weight of dry sample in air = 20 g

B = weight of saturated sample in air  = 22.5 g, fluid density = 1 g/cm3

C = weight of fluid in pore space = B-A = 22.5-20 = 2.5 g

D = Volume of water in pore space = 2.5 gm/1 gm/cc

= 2.5 cc

E = Effective pore volume= 2.5 cc

Bulk volume of rock (Problem 1) = 9.8 cc

=interconnected pore volumeBulk volume

=Effective pore volumeBulk volume

2.59.8 ×100=25.5 per cent

FUNDAMENTALS OF FLUID FLOW IN POROUS MEDIA

نستخدم ال gasoil في الاختبار عادة لكي لا يحدث swelling عند ملامسة الماء العذب لل     shale (Ca)

Homework:

A clean and dry core sample weighing 425 g was 100% saturated with 1.07 specific gravity (ɤ) brine. The new weight is 453 g. The core sample is 12 cm in length and 4 cm in diameter. Calculate the porosity of the rock sample.

Solution:

Petrophysics 4th by Djebbarللجيولوجي

Homework:

Coated sample immersed in water

Weight dry sample in air= 20.0 gm

Weight dry sample coated in paraffin=20.9 gm (Density of paraffin= 0.9 gmcc)

Weight coated sample immersed in water at 40°F= 10.0 gm (Density of water= 1.0 gmcc)

Find bulk volume of rock?

Solution:

Weight of paraffin= 20.9-20.0=0.9 gm

Volume of paraffin=0.90.9=1 cc

Weight of water displaced= 20.9-10.0=10.9 gm

Volume of water displaced =10.91.0=10.9 cc

Volume of water displaced- Volume of paraffin=10.9-1=9.9 cc

Bulk volume of rock= 9.9cc

مصدر 45

Boyle’s Law Porosimeter

تعتبر من الطرق الحديثة لقياس المسامية , وهي طريقة غير إتلافيه (وذلك لعدم استخدام الزئبق) وذلك بعدم تسليط ضغوط عالية (psi 700)  اي الضغط المسلط تقريبا 100 psi)).

الغازات المستخدمة

- الهواء

- Co2

- He غاز الهليوم (وهو من الغازات الخاملة؛ لأنّها غازات غير نشيطة كيميائياً, ويُشار إلى أنّ لزوجة الهيليوم تُساوي صفر تقريباً؛ لذلك فإنّه يمرّ من خلال أصغر وأدق الشقوق ويتسرّب بسهولة من فوّهات الأوعية التي يوضع فيها)

P1×V1=P2×V2

P1×Vref=P2×[Vref+Vcor-Vgra]

P2× Vgra=P2×[Vref+Vcor]- P1×Vref

Vgra=P2×Vref+Vcor-P1×VrefP2

Vgra = Vref+Vcor-p1p2×Vref = Vmat

Example 4

A full diameter core (D=3”, L=6”) is placed in a core chamber of Boyle’s law device, each cell has a volume of 1000cc.

After evacuation cell one is pressured to 50 psig, the valve is opened and the two cells are connected & the final pressure is 28.1 psig.

Calculate the porosity?

P1=50+14.7=64.7 psia

P2=28.1+14.7=42.8 psia

Vb=π/4 D2×L=π/4 ×(9)×6×(2.54cm/inch)3

Vb=965 cc

Vgra = Vref+Vcor-p1p2×Vref = Vmat

Vm=2000-(64.7/42.8)×1000

Vm=488.32

Vp=Vb-Vm= 695-488.32=206.6

Ø=Vp/Vb=206.6/695 =29.73%

A third method is the Boyle’s Law Method. As the name implies, it uses Boyle’s Law for the grain volume determination:

p1V1=p2V2

Equation 3.04

In this method, the core sample is placed into one chamber of an experimental apparatus containing two chambers of known volume connected by a closed valve. An inert gas (helium or nitrogen) is introduced into the chambers at different, but known, pressures. At this point, the total number of moles, nT, in the apparatus can be determined from:

nT=n1+n2

Equation 3.05

PV=nRT

Equation 3.06

pfVTRT=p1V1RT+p2V2RT

Equation 3.07

For isothermal conditions, this reduces to Boyle’s Law:

Example of Boyle’s Law Porosimeter

pfVT=p1V1+p2V2

Equation 3.08

The valve between the chambers is opened and the pressure is allowed to stabilize to the final pressure, pf, and is recorded. If we assume that the core sample was placed into Chamber 1, then we have:

pf(V1+V2−Vg)=p1(V1−Vg)+p2V2

Equation 3.09

# PNG 301: Introduction to Petroleum and Natural Gas Engineering

# 3.2.1: Porosity from Laboratory Measurements

Note: The reservoir rock may generally show large variations in porosity vertically but does not show very great variations in porosity parallel to the bedding planes.

كما لوحظ ذلك من خلال الدراسات المكمنية و النتائج المختبرية وعليه وجدت صيغ معتمدة لحساب معدل المسامية.

These averaging techniques are expressed mathematically in the following forms:

Arithmetic average =∑φ ni

Thickness-weighted average =∑Ø×h∑h

Areal-weighted average =∑A×Ø∑A

Volumetric-weighted average =∑A×h×Ø∑A×h

where

n = total number of core samples

hi = thickness of core sample i or reservoir area i

φi = porosity of core sample i or reservoir area i

Ai = reservoir area i

Example 5

Calculate the arithmetic average and thickness-weighted average from the following measurements:

Indirect methods for measuring porosity, they are well log methods, which is commonly used:

1. The sonic log (it depends that sound travels faster in solid bodies than in liquids).

2. Neutron log ( working principle is that neutrons collide with hydrogen atoms then absorbed).

3. Density log (measures the density difference in the scanned area and then converts it to porosity).

Homework:

A clean and dry core sample weighing 425 g was 100% saturated with 1.07 specific gravity (ɤ) brine. The new weight is 453 g. The core sample is 12 cm in length and 4 cm in diameter. Calculate the porosity of the rock sample.

Solution:

Petrophysics 4th by Djebbarللجيولوجي