Example

اعطي

A sample of reservoir liquid with volume of 400 cc at res. Condition were passed through a separator into a stock tank at atmospheric pressure and 60°F. Volume of liquid in the stock tank was 274 cc a total of 1.21 SCF of gas was relieved.

Calculate the O.F.V.F.

Solution

Bo=vol. at res. cond.vol. at s.c. =400274 =1.46 RB/STB

Shrinkage factor = bo=11.46 = 0.68 STB/RB

اي كل برميل مكمني يعادل 0.68  عند الخزان

وذلك نتيجة تحرر الغاز من الطور السائل

Example

اعطي

Figure below shows the variation in the formation volume factor of the reservoir liquid of the Big Sandy reservoir as a function of pressure at reservoir temperature of 160°F. Because no gas is released from solution when the pressure drops from the initial pressure of 3500 psia to the

bubble-point pressure at 2500 psia, the reservoir fluid remains in a single (liquid) state; however, because liquids are slightly compressible, the FVF increases from 1.310 bbl/STB at 3500 psia to 1.333 bbl/STB at 2500 psia. Below 2500 psia, this liquid expansion continues but is masked by

a much larger effect: the decrease in the liquid volume due to the release of gas from solution. At 1200 psia, the FVF decreases to 1.210 bbl/STB, and at atmospheric pressure and 160°F, the FVF decreases to 1.040 bbl/STB. The coefficient of temperature expansion for the 30°API stock-tank oil of the Big Sandy reservoir is close to 0.0004 per degrees Fahrenheit; therefore, one stock-tank barrel at 60°F will expand to about 1.04 bbl at 160°F, as calculated from

هنا يتم حساب تأثير الحرارة

أي β =Co من معادلة Isothermal Compressibility Coefficient of Crude Oil

VT=V60[1+β(T-60)

VT=1.00[1+0.00040 160-60=1.04 bbl

Where

β (beta) is the temperature coefficient of expansion of the oil = 0.0004.

One obvious implication of the formation volume factor is that for every 1.310 bbl of reservoir liquid in the Big Sandy reservoir, only 1.000 bbl, or 76.3%, can reach the stock tank.

X= 76.3%

Example

اعطي

Visual conception of the change in single-phase and in two-phase formation volume factors for the Big Sandy reservoir fluid.

(A) shows a cylinder fitted with a piston that initially contains 1.310 bbl of the initial reservoir fluid (liquid) at the initial pressure of 3500 psia and 160°F. As the piston is withdrawn, the volume increases and the pressure consequently must decrease. At 2500 psia, which is the bubble-point pressure, the liquid volume has expanded to 1.333 bbl. Below 2500 psia, a gas phase appears and continues to grow as the pressure declines, owing to the release of gas from solution and the expansion of gas already released; conversely, the liquid phase shrinks because of loss of solution gas to 1.210 bbl at 1200 psia. At 1200 psia and 160°F, the liberated gas has a deviation factor of 0.890, and therefore the gas volume factor with reference to standard conditions of 14.7 psia and 60°F is

Bg=VP,TVS.C=ZnRTP×Vs.c   5.614 ft3  / bbl

Since Vs.c. =379.4 scf/ib-mol

Figure below shows an initial solution gas of 567 SCF/STB and, at 1200 psia, 337 SCF/STB, the difference of 230 SCF being the gas liberated down to 1200 psia. The volume of these 230 SCF is

Rsi-RS=567-337=230  SCF

Vg=gas volume at reservoir condition=230×0.01300=2.990 ft3

This free gas volume, 2.990 ft3 or 0.533 bbl, plus the liquid volume, 1.210 bbl, is the total FVF or 1.743 bbl/STB—the two-phase volume factor at 1200 psia. It may also be obtained by Eq. (2.28) as

Figure 2.10 (C) shows these separate and total volumes at 1200 psia. At 14.7 psia and 160°F (D), the gas volume has increased to 676 ft3 and the oil volume has decreased to 1.040 bbl. The total liberated gas volume, 676 ft3 at 160°F and 14.7 psia, is converted to standard cubic feet at 60°F and 14.7 psia using the ideal gas law, producing 567 SCF/STB as shown in (E). Correspondingly, 1.040 bbl at 160°F is converted to stock-tank conditions of 60°F as shown in Eq. (2.28) to give 1.000 STB, also shown in (E).