Lab3:

Centrifuge

A centrifuge is a device used to separate components of a mixture on the basis of their size, density, the viscosity of the medium, and the rotor speed.

Principle

- In a solution, particles whose density is higher than that of the solvent sink (sediment), and particles that are lighter than it floats to the top .

• The greater the difference in density, the faster they move

• If there is no difference in density (isopycnic conditions), the particles stay steady

• A centrifuge is a piece of equipment that puts an object in rotation around a fixed axis (spins it in a circle), applying a potentially strong force perpendicular to the axis of spin (outward)

• The centrifuge works using the sedimentation principle, where the centripetal acceleration causes denser substances and particles to move outward in the radial direction

• At the same time, objects that are less dense are displaced and move to the centerCentrifuge

• The centrifuge is commonly used in laboratories for the separation of biological molecules from a crude extract.

• In a centrifuge, the sample is kept in a rotor that is rotated about a fixed point (axis), resulting in strong force perpendicular to the axis

- Parts of a Centrifuge Centrifuge consist of:

- operation time control (timer)

- rotation speed control (in some centrifuges)

- temperature control (in refrigerated centrifuges)

- vibration control (safety mechanism) and brake system.

- Refrigeration system (in refrigerated centrifuges).

- Vacuum system ).

- Base.

- . Cover.

- Casing.

- Electric motor.

- Rotor to hold tubes Drive shaft

- Drive shaft

.

Applications of Centrifugation

• To separate two miscible substances

• To analyze the hydrodynamic properties of macromolecules

• Purification of mammalian cells

• Fractionation of subcellular organelles (including membranes/membrane fractions) Fractionation of membrane vesicles • Separating chalk powder from water

• Removing fat from milk to produce skimmed milk

• Separating particles from an air-flow using cyclonic separation

• The clarification and stabilization of wine

• Separation of urine components and blood components in forensic and research laboratoriesAids in the separation of proteins using purification techniques such as salting out, e.g. ammonium sulfate precipitation. .